LBT PROJECT

TELESCOPE ROTATING ENCLOSURE

TECHNICAL REPORT

- ROTATION BOGIES AND DRIVES -

W. Gallieni

- INDEX -

- 1 - 	APPLICABLE DOCUMENTS -

- 2 -	DESIGN LOADS DEFINITION -

- 3 - 	ENCLOSURE INERTIA -

- 4 -	WHEEL AND RAIL SELECTION -

- 5 - 	DRIVES DESIGN -

- 6 - 	HORIZONTAL ROLLERS -

- 7 - 	INTERFACES BETWEEN BOGIES AND BUILDING -

- 1 - APPLICABLE DOCUMENTS -

- 1.1 - LBT Telescope Specification - UA - 95 -XX - and the 12 Jul 1995 email of 	    	 John Hill on the subject - ( for rotation speed and acceleration and for wind 	 	 speed ) -

- 1.2 - M3 Eng. MEETING REPORT # 050 dated June 4/5, 1996 - revised and 	distributed by John Hill on 7/17/96 - ( for max. design loads on the bogies) -

- 1.3 - M3 Eng. - WEIGHT BUDGET SUMMARY dated 31 May, 1996 - as distributed 	during the Jun 4/6, 1996 Arcetri Meeting. The enclosed “CALCULATION OF 	WIND FORCES ON BUILDING STRUCTURE” are the values to be used for 	defining the “wind torsion moment at CL pier” for the selection of driving gears 	and 	motors.

- 1.4 - LIST OF ANNEXED DRAWINGS -

341A000/B - Rear and front bogies assembly -

341A001/B - Horizontal guide rollers. -

341A002/B - Drive wheel assembly -

341A003/A - Stow pin assembly -

341A004/B - Leg bottom flange connection to bogie -

- 2 - DESIGN LOADS DEFINITION -

- 2.1 - Wind velocity to be considered for all the rotating building design conditions -

- 2.1.1 - Max. observing wind speed			50 mph    =      80 Km / h

( means the highest wind speed in which the building can rotate with open 	 	      shutters) -

- 2.1.2 - Max. operating wind speed			75 mph    =    120 Km / h

( means the highest wind speed in which the building can rotate with closed 	 	      shutters ) -

- 2.1.3 - Survival wind speed open				90 mph    =    150 Km / h

- 2.1.4 - Survival wind speed closed		                  135 mph    =    225 Km / h

- 2.2 - Wind Forces and Moments generated in the above conditions -

For the above conditions are summarised the figures taken from the M3 	Calculation Sheets, deriving the figures for different wind speed with the rule of 	pressure being proportional to the square value of the wind speed.

- 2.2.1 - Observing max. wind speed, (80 Km / h), open shutter, wind perpendicular 	        to shutter -

TIPPING TOTAL WIND FORCE -	 	    2388 * (50:90)² =          737 KN

MOMENT at top of rail      			54839 * (50:90)² = 16925 KN.m

TORSION MOMENT at C/L pier                                                    	        =                      0 KN.m

- 2.2.2 - Observing max. wind speed, (80 Km / h), open shutter, wind at 45 degrees 	        to shutter -

TIPPING TOTAL WIND FORCE -		    2643 * (50:90)² =          816 KN

MOMENT at top of rail      			58794 * (50:90)² = 18146 KN.m

TORSION MOMENT at C/L pier                            12080 * (50:90)² =      3728 KN.m

- 2.2.3 - Wind speed 120 Km / h, closed shutter, wind parallel to shutter -

TIPPING TOTAL WIND FORCE -		                                                                1230 KN

MOMENT at top of rail      			                                                            27309 KN.m

TORSION MOMENT at C/L pier                                                                                            1499 KN.

- 2.2.4 - Wind speed 120 Km / h, closed shutter, wind at 45 degrees to shutter -

TIPPING TOTAL WIND FORCE -		                                                                1585 KN

MOMENT at top of rail      			                                                            34468 KN.m

TORSION MOMENT at C/L pier                                                                                            2682 KN.m

- 2.2.5 - Wind speed 225 Km / h, closed shutter, wind parallel to shutter -

TIPPING TOTAL WIND FORCE -		                                                                3148 KN

MOMENT at top of rail      			                                                            69910 KN.m

TORSION MOMENT at C/L pier                                                                                            3838 KN.m

- 2.2.6 - Wind gusts at 225 Km / h, closed shutter, wind at 45 degrees to shutter -

TIPPING TOTAL WIND FORCE -		                                                                4058 KN

MOMENT at top of rail      			                                                            88237 KN.m

TORSION MOMENT at C/L pier                                                                                            6867 KN.m

- 2.3 - Total mass of the enclosure and its distribution over the bogies and 	 	 	 meteorological overloads.

- 2.3.1 - Telescope in operation, crane in the rear storage position, values as per M3 	        Meeting Report # 50.

Back Bogie			Front Bogie

Dead Load				2530 KN			4190 KN

Astronomers dead load	    167 KN			    324 KN

80 Km / h wind load	                ± 667 KN		                ± 667 KN

-----------			-----------

Max. total	3364 KN			5181 KN

Vertical adopted design Loads 3435 KN		5200 KN

N.B.: the dead load of 13440 KN is the value resulting from the Weight Budget Summary, while the wind overload is an average figure between the 520 KN with wind in perpendicular direction and the 785 KN with the wind at 45 degrees direction.

Similarly, the design horizontal wind force with the telescope in operation is defined in            ( 737+816 ) / 2 = 776,5 KN -

- 2.3.2 - Survival conditions, closed shutter, 225 Km / h survival wind gusts.

Back Bogie			Front Bogie

Dead Load				2530 KN			4190 KN

Astronomers dead load	    167 KN			    324 KN

225 Km / h wind load	            ± 3845 KN		            ± 3845 KN

50% snow load			    245 KN			    490 KN

-----------			-----------

Max. total	 6787 KN		                  8849 KN

Allowable reduction =

75% of load            		                        	5090 KN		6637 KN

Vertical adopted design loads	5100 KN		6870 KN

Total Horizontal wind load                	3924 x 0,75 = 2943 KN

Earthquake lateral load                          	2943 KN

- 2.4 - Wind torsion moments at the rotation axis for drives design -

- 2.4.1 - Observing condition, wind 80 Km / h at 45 degrees to shutter direction -

Mt = 3728 KN.m

- 2.4.2 - Max. operating condition, shutter closed, wind 120 Km / h at 45 degrees to 	            shutter direction -

Mt = 2682 KN.m

N.B. : Torque smaller than the one in observing conditions with open shutter, so this one is not a critical condition to be verified.

- 3 - ENCLOSURE INERTIA -

Using the building dimensions of the June 96 Arcetri Meeting version and the masses of the 31 May 1996 Weight Budget Summary of M3, the following figures of moment of Inertia has been calculated :

- 3.1 - Open Shutter, Total Mass 13440 KN 		J = 3,4706.E8 Kg.m²

- 3.2 - Closed Shutter,      “              “                      “      			J = 2,9161.E8 Kg.m²

N.B. : the values have been calculated reducing the assembly to a number of subassemblies of simplified shape, so the values, sure valid for a correct choice of the drives, should be verified with the final FEA model by M3.

- 4 - WHEEL AND RAIL SELECTION -

- 4.1 - Requested Performances -

Max. angular velocity						1,5 degrees. sec¯¹

Max. angular acceleration					0,3 degrees. sec¯²

Max. azimuth rotation						± 270 degrees from South

Max. operating wind speed - Shutter open - 	    	80 Km / h

Max. operating wind speed - shutter closed - 		120 Km / h

Clearance on telescope rotation				± 2 degrees

- 4.2 - Wheel selection -

- 4.2.1 - Wheel mean load in operation :

Considering the uneven distribution of loads between the front and the rear bogies, to use wheels of the same dimension having approximately the same max. load, has been chosen four wheel bogies for the rear corners and six wheel bogies for the front ones; the rear wheels have a max. load of 3435 / 4 = 858,75 KN and for the front ones the figure is 5200 / 6 = 866,66 KN, with a min. value of 641,6 KN, figures that become the dimensioning for all the bogies.

Pm = = = 791,6 KN

Selected wheel diameter					D = 100 cm

Selected rail useful width					 b =      13,4 cm

Mean rail diameter						Dr=      23 m

Wheel max. speed at w=1,5 degrees per second      	= 5,75 rpm

- Material : alloy steel, through hardened min. 36 HRC

- Pamm = 650 N / cm²

- c1 - Velocity coefficient = 1,16 for 5,75 rpm

- c2 - Mechanical Group 8 - = 0,8

1 x c2

590 £ 603

Wheel with revolving axis, supported by roller bearings, without ribs ( horizontal guide rollers ), has a conservative value of resistance to movement of 3 N / KN -

- 5 - DRIVES DESIGN -

- 5.1 - Demanded torque at the rotation axis, and tangential forces at the rail top surface.

- 5.1.1 - Tangential force due to resistance to movement :

Fw = 14420 x 3 = 43260 N

The corresponding torque at the rotation axis is :

MtFw = 43260 x 11,5 = 0,49749.E6 Nm

- 5.1.2 - Torque for accelerating the building inertia with open shutter :

w* = = 5,24.E-3 1/sec²

Mtacc = J x w* = 3,4706.E8 x 5,24.E-3 = 1,8186.E6 Nm

The corresponding tangential force is :

Facc =

- 5.1.3 - Torque due to the 80 Km /h wind :

MW80 = 3,728.E6    Nm

The corresponding tangential force is :

Fw80 =

- 5.2 - Motor and gear selection -

The selected design is the one that foresees two driving units for each corner, thus a total of eight gear/motor units that represent the 50% of the rear wheels and the 33% of the front ones, assuring a good safety coefficient when we have to verify the transmissible tangential driving force with a 0,14 friction coefficient between rail and wheel.

To have the possibility of selecting four poles AC motors VFD controlled, we have increased the reduction ratio of the gears ( including one more reduction couple) to 1:315 compared to the 1:200 of the 1994 preliminary design, so that to the max. wheel velocity of 5,75 rpm correspond a motor speed of about 1880 rpm, i.e. the nominal velocity at

60 HZ.

It is not in the scope of this report to select the type of motor - and the relevant drives - to be used, but, both in the case of AC or DC motors, all must be equipped with spring operated disc brakes and very likely with force ventilation to take into account the very low speed to be developed during the tracking, also if shall be introduced the control taking care of the ±2 degrees clearance position with respect to the telescope.

- 5.2.1 - Nominal motor velocity 		1880 RPM = 188,5    rad/sec

Nominal wheel slew velocity	1800/315      =    5,714 rpm

- 5.2.2 - Total transmission ratio between motor and building axis :

t =

- 5.2.3 - Estimated efficiency of the gear 		h = 0,88

- 5.2.4 - Demanded torque reduced to the motor axis :

For the resistance to movement of the wheels :

CWR= 497490/(8 * 7245 * 0,88) = 9,75 Nm

For accelerating the inertia with open shutter :

Cacc = 1818600/(8 * 7245 * 0,88) = 35,65 Nm

For withstanding the 80 Km / h wind :

Cw80 = 3728000/(8 * 7245 * 0,88) = 73,09 Nm

The wind torque is the prevailing factor in defining the size of the motors and gears, much more important than in the previous 1994 evaluation; remaining in the field of AC motors VFD regulated, I should select a 12,5 KW, 4 poles, Cm / Cn = 2,5 controlled with an inverter able to supply at 1 HZ a continuous torque of 86 Nm (about 1,3 the nominal torque), and a peak torque of 120 Nm between 1 an 60 HZ (about 1,8 times the nominal torque).

The gear is requested to transmit the nominal torque, reduced at the low speed shaft - that is the wheel axis -, that is Cn = 66,31.315 = 20900 Nm, that becomes about 27200 Nm when tracking against the 80 Km / h wind and about 37600 Nm when accelerating in the above condition; The previously selected 315 size of our standard supplier has a nominal torque of 40000 Nm in the four gear train configuration, and can be confirmed.

- 6 - HORIZONTAL ROLLERS -

- 6.1 - Loads in operation with open shutter and 80 Km/h wind.

Rollers estimated diameter 		D = 400 mm

Rollers nominal velocity		n = 23    *    p / 0,4    * p x 0,25= 14,375 rpm

When the building is in movement we consider that the supporting wheels cannot absorb any important transversal force, so the whole wind force is distributed on two couples of opposite rollers, that gives a max. value each roller of :

Hw = 776,5 / 4 = 194 KN

Assuming for the roller the same material of the wheels and considering the

c2 velocity coefficient = 1,1 for the velocity of 14 rpm, fixing the useful contact width in

7,4 cm, we verify :

Pm = 194000. 2/3 =129400 N

- 6.2 - Loads in survival conditions -

The most unfavourable condition is the 225 Km / h wind gusts at 45 degrees to shutters. without the snow surcharge and Astronomers dead load.

We can calculate that the friction between rail and wheels absorbs horizontal forces, assuming a conservative coefficient of friction m=0,2, as follows :

Front bogie in wind direction 	Fad = 4190 * 0,2 = 838 KN

Front bogie parallel to wind	Fad = 4190 * 1/3.0,2+4190 * 2/3 * 3/1000 = 288 KN

Rear bogie in wind direction	Fad    = 2530 * 0,2 = 506 KN

Rear bogie parallel to wind	Fad    = 2530 * ½ * 0,2+2530 * ½ * 3/1000 = 257 KN

Total absorbed lateral force	Fad = 1889 KN

The lateral force absorbed by each of the two couples of opposite rollers is :

FL max =  KN 	having reduced to the 75% the wind 								gusts load

In conclusion, every horizontal roller is to be verified in the survival condition for a max. force of 527/2 = 263,5 KN .

- 7 - INTERFACES BETWEEN BOGIES AND BUILDING -

- 7.1 - First point to be evidenced is the position of the C/L of the upper flange of the bogies that, as a consequence of the angle of convergence of the complete bogie to compensate the choice of adopting a flat head of rail design, are displaced of a sensible dimension toward the rotation axis compared to the main building beams C/L positioned at 16263,5x16263,5 scheme distance.

As can be seen on the preliminary assembly drawings enclosed for comments, the difference is small compared to the dimensions of the frame and could be accommodated properly displacing the fixation bolts scheme in each corner, taking into account that has been decided to design both the 4 wheels and the 6 wheels bogies with the interface top plate level at the same figure, so to eliminate differences in length of the enclosure legs.

In our design we have foreseen to compensate the conical wheels semiangle with the machining of the centre connection frame, so that the interface with the building leg is horizontal; in the M3 design we suggest introducing a couple of opposite shimming plates for each flange, so to have the possibility of a final adjustment of the bogie inclination in respect to the vertical.

- 7.2 - Second point is the structural dimensioning of the connection flange and consequently the building leg.

When in observing condition with open shutter and 80 Km / h wind at 45 degrees, if we consider the four wheels back bogie unloaded by the wind component, we have the following situation :

Vertical load = 2530+167-667 = 2030 KN

Horizontal load = 776,5 / 2 = 388,25 KN

The bending moment at the interface flange of 388,25 x 2,27 = 881,32 KN.m produces a tensile force on the 50% of the bolted joint of 881,32 / 1,5 = 587,5 KN, value still lower than the 50% of the vertical load, so the bolts must transmit the shear load only and do not become in tension.

One vertical flange of the centre connection frame has to transmit a vertical compression load of 2030 / 2 + 587,5 = 1602,5 KN.

In each one of the main symmetry axis of the leg we introduce a bending moment of 881,32 / 2 * sin 45º = 623,2 KN.m, moment that reaches their max. value at the level of the machinery floor axis, i.e. about 2,6 m over the interface flange, that means about a value of 1340 KN.m.

-7.2.1 - When we consider the same corner in the survival condition we must take into account that the vertical component due to the wind tipping moment unloads completely the dead load component, so the only force insisting on the bogie is the lateral component of the wind force, i.e. 1017,5 KN in the most conservative hypothesis - see the point 6.2 -.

In the bolts of the half interface flange we must transmit a tensile force of

T = 1017,5 * 2,27 / 1,5 = 1540 KN    , that means 1540 / 14 = 110 KN per each bolt to be combined with the shear force.

In each of the main axes of the most stressed section of the building leg we have a bending moment of :

Mf = 1017,5 * 4,87 / 2 * sin45º = 3504 KN.m

Under these extreme conditions it is not only the stress to be verified, but also the horizontal displacement consequent to the legs deflection, to avoid problems in the gaps between enclosure and telescope.

- 7.2.2 - Shear transmission between leg and bogie -

The centre frame of the bogies is foreseen as a single piece    U shaped and open at the top, because to easier the construction, machining and assembly of the bogie; the reason is that the closure of the structure can be obtained with the end flange of the building leg.

The shear effort between leg and bogie could be transmitted shaping properly the bottom flange of the leg, either machining it as per the enclosed conceptual sketch, or welding after the correct alignment on site a couple of additional plates.

In the second hypothesis we could increase    of 100 mm or more the height of the centre frame to give more free space to easier the welding operation.

-7.2.3 - Bogies verification under survival conditions, -

When we consider the 6 wheel bogie in the opposite position to the one verified at the point 7.2.1, i.e. the bogie that is overloaded by the vertical component of the wind tipping moment, the lateral force of 1017,5 KN    transmitted by shear through the flanges and that is resisted by the rail through the contact of the horizontal rollers, must be increased by the 838 + 506 = 1344 KN transmitted by friction to the rail top surface through the wheel flanges in contact, forces that generate torsion moment and horizontal bending moment in the single bogie frame and that at the end increase the bending moment in the building leg to the figure of :

Mf = 2943 * 2,60 = 7651,8 KN.m at the bottom flange

Mf max. leg = 2943 * 4,87/ 2 * sin45 = 10134,5 KN.m

- 7.2.4 - Stow pin dwg b341a003 -

The stow pin is not dimensioned for absorbing any fraction of the horizontal lateral load due to wind or earthquake, because all are verified transmitted to the rail by friction or by contact of the horizontal guide rollers. the stow pin is intended to be a safety interlock with the rotation movement when the building is positioned in the auxiliary building direction with the maintenance hatch open.

The stow pin axis is positioned with an offset of 230 mm in respect to the bogie axis, so to have the possibility of locking the building in the direction of the auxiliary building axis avoiding the coincidence with the axis of the runway and rail joint.

- 7.2.5 - Bogies and drives mass -

For the purpose of a correct final verification of the rotating building weight and to prepare sufficient information for the call for tender for the construction of the units, has been performed an evaluation of the weight of the single components of the mechanism, obtaining the following values :

Four wheel bogie                		22220 kg each,            total for two units 44440 kg

Six wheel bogie			32870 kg each,	    total for two units    65740 kg

TOTAL WEIGHT OF THE MECHANISM		    ~ 110 000 kg

Doc_info_start

Title:Rotating Bogies and Drives

Document Type: Technical Report

Source:ADS Italia

Issued by:W.Gallieni

Date_of_Issue:01/14/97

Revised by: J. Hill

Date_of_Revision: 06/12/97

Checked by:W.Gallieni

Date_of_Check:04-02-97

Accepted by:

Date_of_Acceptance:

Released by:

Date_of_Release:

File Type:MS-WORD 6

Local Name:

Category:Rotating Enclosure

Sub-Category:General Rotating Enclosure

Assembly:Technical Documentation

Sub-Assembly:Rotating Bogies and Drives

Part Name:

CAN designation:301a002

Revision:C

Doc_info_end