LBT PROJECT

TELESCOPE ROTATING ENCLOSURE

T E C H N I C A L    S P E C I F I C A T I O N

ROTATION BOGIES AND DRIVES

W. Gallieni

INDEX:

1.INTRODUCTION.

2. TECHNICAL DATA AND DRAWINGS.

3. SUPPLY DESCRIPTION.

4. CONSTRUCTION AND ASSEMBLING PROCEDURE.

5. TESTING.

1 . INTRODUCTION -

The construction of the rotation bogies and drives is the main mechanical part of the

LBT Project Telescope Rotating Enclosure.

To simplify the construction and machining of the circular rail (23 m in diameter), it has been designed as a flat top rail, and is necessary displacing the centre convergence point of the wheels axis at the same level of the rail top, inclining the bogies and wheels axis of half the cone in which are machined the wheels contact surfaces.

This is the normal solution used in the design of the big radio telescopes azimuth axis rotation drives, and requires only an accurate procedure during assembling and pre-alignment in shop and at the final assembling at the erection Site.

The connection flanges between the upper surface of the main equalising beams and the building legs, also if have been located at the same level to easier the interfaces design, are the detail that needs a very good agreement and understanding with the building Manufacturer, because of the decision of how transmitting the horizontal shear forces and the positioning procedure that defines the tolerances in the fixation bolts bores.

For reasons of standardisation of the electro-mechanical components present in the various subsystems of the rotating enclosure, the P.O. shall order and delivery at his charge to the bogies Manufacturer the nine motor - reducer units (eight to be assembled and one spare).

2. TECHNICAL DATA AND DRAWINGS-

2.1. General technical data-

- Circular track nominal diameter				23,00 m

-Rail useful width							140,0 mm

- Circular track max. runout						± 2 mm

- Enclosure nominal rotation velocity				w = 1,5 degree/sec

= 0,25 rpm

- wheel ( 100 cm diameter) nominal speed			        = 5,75 rpm

- Wheels total number						        = 20

- Motors n. 8 x 12,5 KW / 4 poles ( 2 each bogie)

-estimated weight for one four wheels bogie 				 ~    22200 kg

-estimated weight for one six wheels bogie				    ~ 32900 kg

- estimated weight of the materials to be supplied, excluding the

driving units supplied by P.O.						    ~ 97000 kg

To help the Manufacturer in the preparation of the shop drawings, the Technical Specification shall be integrated with the Technical Report 301a002 resuming all the loads transmitted on the wheels in the various working conditions.

2.3. Drawings. The following drawings are enclosed together with this specification.

Dwg. 341A000/B		REAR AND FRONT BOGIES ASSEMBLY      -

Dwg. 341A001/B		HORIZONTAL GUIDE ROLLERS

Dwg. 341A002/B		DRIVE WHEEL ASSEMBLY

Dwg. 341A003/A		STOW PIN ASSEMBLY

Dwg. 341A004/B		LEG BOTTOM    FLANGE CONNECTION TO BOGIE

Dwg. 341A009/A		DRIVING UNITS TORSION REACTION ARM

TYPICAL ASSEMBLY

Dwg.    341A010/A	                      TORSION REACTION ARM - TYPICAL DETAIL

Dwg.    341a006/B		ENCL. DRIVE MECH. - PLANT GENERAL

Dwg.    341a007/B		ENCL. DRIVE MECH. - LATERAL SECTION

3 .SUPPLY DESCRIPTION.

3.1 - Shop Drawings, planning and technical documentation.

This is the first activity to be performed by the Manufacturer and shall be performed according what specified in the “Statement of Work”, attachment A of the RFP.

3.2 - Supply Limits -

The supply includes the construction and the machining of the steel structure of the various equalising beams and wheels supports, and the supply and integration of the wheels, articulation axis, bearings and all the necessary mechanical accessories. Also included in this supply is the integration of the eight motor/ reducers driving units that shall be supplied by the P.O.

The Supplier shall preassemble the four bogies groups - n. 2 four wheel units and n.2 six wheel units -, checking their radial alignment and convergence toward the centre of the rail circular track. In each of all the main articulation shafts of the group will be foreseen a through centre hole, about 20 mm diameter with the attachments for the collimator for the convergence verification.

The gear / motor units supplied by the P.O. shall be connected, at the care of the Manufacturer, to the relevant bogie frame through a torsion reaction arm, the typical assembly drawing is included in the tender documents.

The Manufacturer of the Bogies shall supply to the care of the Manufacturer of the Rotating Enclosure a set of four machined plates. These steel plates, of thickness TBD, shall act as templates for the connection borings between the top part of the central frame of the bogies and the lower part of the building legs. After assembly of the Rotating Enclosure steel frame on the bogies, these plates shall also be machinable as shim plates to make final adjustment of the angle of the connections between the building legs and the bogies.

With reference to the possible erection and Site alignment procedure - see dwg.341a006 and 341a007 -, the Manufacturer shall include in his supply also the eight adjustable temporary anchoring tie-rods to the telescope pier.

3.3. Surface protection and Painting.

After the stress relieving heat treatment and sand blasting SA 2.5 , all the surface of the bogies structure, must be protected with Primer (125 micron thickness), before machining.

After machining and careful cleaning must be applied a second layer of epoxy type paint ( 100 micron thickness ) on all the inside and    outside surfaces

3.4. Packing.

The packing of the units must be described in detail in the supply offer foreseeing that the same shall be shipped by sea    (in the case of European Supplier ) and than transported by truck to the staging area in Safford Arizona. The bogies may be stored outdoors in the staging area for several months before they are installed on the mountain.

3.5. General rules.

The supply must comply    with the 89 / 392 / CEE rules.

- 4 - CONSTRUCTION AND ASSEMBLING PROCEDURE -

4.1. Planning -

The Program for the Construction shall include a complete description of the construction, quality control and machining procedures, being this chapter one of the most important points for the evaluation of the offer and selecting the Manufacturer.

For the two wheel bogies and equalising beams steel structure it must be evidenced the welding procedures standard for the Manufacturer, the type of quality controls adopted, the stress relieving thermal treatments the structures shall be submitted before the machining; it is requested to evidence the machining procedure foreseen to obtain the specified convergence of all axes of the wheels and articulation toward the circular track centre.

A description of the preassembly verification of each 4 and 6 wheels unit in the workshop, demonstrating the radial alignment of all the wheels on the 11500 mm radius, and at the same time the coherent fixation of each two wheels bogie in respect to the relevant    main articulation beam.

5. TESTING.

All the parts that compose the supply must be submitted to acceptance tests in the Manufacturer workshop and certified, namely ;

Functional test of each driving wheel on his bogie, after the correct integration of the motor/gear unit supplied by the P.O.,    and the assembling and locking it to the bogie structure through its torsion reaction arm.

Verification of the correct alignment and convergence of the wheels in each bogie and of every group toward the centre of the circular track.

The interface templates between the building legs and the top plates of the main articulation beams shall be part of a separate agreement and acceptance procedure agreed with both the bogies and building Manufacturers.

Doc_info_start

Title: Rotation Bogies and Drives

Document Type: TECHNICAL SPECIFICATION

Source:ADS Italia

Issued by:W.Gallieni

Date_of_Issue:06/05/97

Revised by:

Date_of_Revision:

Checked by: W.Gallieni

Date_of_Check:06-05-97

Accepted by:

Date_of_Acceptance:

Released by:

Date_of_Release:

File Type: MS-WORD 6

Local Name:

Category: Rotating Enclosure

Sub-Category: General Rotating Enclosure

Assembly: Technical Documentation

Sub-Assembly: Rotation Bogies and Drives

Part Name:

CAN designation:301a003

Revision: D

Doc_info_end