LBT PROJECT

TECHNICAL SPECIFICATION

DEROTATORS STRUCTURE SUPPORTS

| - Document No.: | - 401e031 |
| --- | --- |
| - Issue: | - b |
| - Date: | - January 19th, 1998 |
| - Prepared by: | - Gianpietro Marchiori/EIE |
| - Approved by: | - Gianpietro Marchiori/EIE |

INDEX

1.0 SCOPE OF WORK

2.0 APPLICABLE DOCUMENTS

3.0 ITEM DESCRIPTION

4.0 INTERFACE DEFINITIONS

5.0 PERFORMANCE CHARACTERISTICS

6.0 RELIABILITY, MAINTAINABILITY REQUIREMENTS

7.0 ENVIRONMENTAL CONDITIONS

8.0 DESIGN AND CONSTRUCTION REQUIREMENTS

8.1 General Dimensions and Mass

8.2 Materials

8.3 Class of Welding

8.4 General Tolerances

8.5 Thermal Treatments

8.6 Tightening Torque of Bolts and Screws

8.7 Surface Treatments

8.8 Special Manufacturing Instructions

9.0 PRE-ASSEMBLY

10.0 PACKING AND TRANSPORT

11.0 ACCEPTANCE TESTS

12.0 SPARE PARTS	 17

ANNEX A

# 1.0	SCOPE OF WORK

The DEROTATORS STRUCTURE is the element composing the LBT Project.

The aim of the LBT (Large Binocular Telescope) Project is to realise a telescope having two mirrors of 8.4 m diameter, to be installed at Mount Graham in Arizona (ref. dwg. No. 407e001).

The DEROTATORS STRUCTURE represents a part of the elevation structure of the telescope.

The DEROTATORS STRUCTURE (L & R) is the support of the derotators instruments and our accessories. This instruments are installed between the two M1 cells, and the DEROTATORS STRUCTURE forms the central structure of the elevation structure.

They are made up of steel structure machined elements, assembled each other by means of bolts.

The manufacturer shall receive, at the charge of the P.O., the six bearings supporting the derotator instrument, the four cable chain support structures, the driving units for the above groups, the gear and pinions driving for the rotator/instrument and the pinion for the cable chain support, all these items shall be pre-assembled and tested in rotation.

The electronic equipment needed to drive the motors shall be supplied by the P.O.

The supply of the DEROTATORS STRUCTURE includes:

- execution of all workshop drawings and Part Lists, based upon the design drawings included in the Request for Proposal;

- preparation of the Manufacturing Plan;

- selection and purchase of all materials, components, etc.

- preparation of all procedures and methods to execute the critical operations of the manufacturing;

- fabrication of all components;

- fabrication and/or purchase of all tools, necessary for the manufacturing and/or assembly;

- painting or surface treatment;

- performance of checks and tests;

- drawing up of the inspection and check documentation.

# 2.0	APPLICABLE DOCUMENTS

| Title | Document |
| --- | --- |
| - Drawings: Elevation StructureCentral Structure/Rotators Structure/General View Elevation Structure/Central Structure/ Rotators Structure/Orthogonal View Elevation Structure/Central Structure/ Rotators Structure/Plan View Elevation Structure/Central Structure/ Rotators Structure/Rotator Supp. Lower Back Beam Elevation Structure/Central structure/Derotators Structure/General Assembly ‘L' Elevation Structure/Central Structure/Derotators Structure/Lower Structure ‘L’ Elevation Structure/Central Structure/Derotators Structure/Upper Structure SH 1/3 ‘L’ Elevation Structure/Central Structure/Derotators Structure/Upper Structure S 2/3 ‘L’ Elevation Structure/Central Structure/Derotators Structure/Connected Plate for C-Ring ‘L’ Elevation Structure/Central Structure/Derotators Structure/Drag Strut Internal Back ‘L’ Elevation Structure/Central Structure/Derotators Structure /Drag Strut External Back ‘L’ Elevation Structure/Central Structure/Derotators Structure/Drag Strut Central ‘L’ Elevation Structure/Central Structure/Derotators Structure/Upper Structure SH 3/3 ‘L’ Elevation Structure/Central Structure/Derotators Structure/Connection Strut Frontal Elevation Structure/Central Structure/Derotators Structure/Connection Strut Back Elevation Structure/Central Structure/Derotators Structure/Drag Strut Frontal ‘R’ Elevation Structure/Central Structure/Derotators Structure/Shims ‘L’ - Interface Drawings: Elevation Structure/Rolling Sectors/General Assembly of C-Ring ‘L’ Elevation Structure/Rolling Sectors/Supporting Arm ‘L’ of Spiders Elevation Structure/Rolling Sectors/General Assembly of C-Ring ‘R’ Elevation Structure/Rolling Sectors/Supporting Arm ‘R’ of Spiders Elevation Structure/Horizontal Bracing/External Nodes/Back Assembly Side ‘L’ Elevation Structure/Horizontal Bracing/Lateral Back Beam Side ‘L’ sh 1/2/Sheet 1/2 Elevation Structure/Horizontal Bracing/Front Beam/Sheet 1/3 Elevation Structure/Horizontal Bracing /Back Beam/Sheet 1/2 Elevation Structure/Horizontal Bracing/ /Rotators Structure Sheet 1/5 Telescope Auxiliaries/Instrument Rotators/Rotator Utilities/Combined Focus Rotator/Section E-E Detail CW Telescope Auxiliaries/Instrument Rotators/Instrument Rotators General/Plan View Nasmyth Instruments Instruments Rotators General//Centre Area Assembly/Drives Units Details Instruments Rotators General/Derotators and Fixed Flange Adaptive/Optics Obstruction/Interface Detail Instruments Rotators General/Instruments Flange/Interfaces Detail Instruments Rotators General /Fixed Gravity Instruments Interfaces/With Fixation Bearing Supporting Instruments Rotators /Instrument Rotators General/Ball Bearing for Nasmyth and Combined Focus Instruments Rotators General /Ball Bearing for Cable Wrap for Nasmyth and Combined Focus Rotator | 513e000 513e002 513e003 513e005 513e006 513e007 513e008 513e009 513e010 513e011 513e012 513e013 513e014 513e015 513e016 513e017 513e018 514e004 514e006 514e023 514e034 512e005 512e011 512e039 512e042 512e045 676a001 670a025 670a026 670a030 670a031 670a032 670a021 670a023 |

# 3.0	ITEM DESCRIPTION

The DEROTATORS STRUCTURE (see dwg. No. 513e000), side L and R, are made up of the following elements:

- No.1+1 LOWER STRUCTURE (L & R)

- No. 1+1 UPPER STRUCTURE (L & R)

- No.1+1 CONNECTED PLATE FOR C-RING (L & R)

- No.1+1 DRAG STRUT INTERNAL BACK (L , R)

- No.1+1 DRAG STRUT EXTERNAL BACK (L & R)

- No.2 DRAG STRUT CENTRAL

- No.1CONNECTION STRUT FRONTAL

- No.1CONNECTION STRUT BACK

- No.1+1 DRAG STRUT FRONTAL (L & R)

- various bolts and accessories

The DEROTATORS STRUCTURE is a steel structural work made up of plates and beam with different thickness, cut, finished and assembled by means of welding.

This element is composed by two DEROTATORS STRUCTURE SUPPORTS (L & R) that will be assembled and machined in accordance to the tolerance indicated in the drawings and in chapter 8.0.

The DEROTATORS STRUCTURE has a structural function to support the instrument derotators and our accessories.

# 4.0	INTERFACE DEFINITIONS

The DEROTATORS STRUCTURE  (see dwg. No. 513e006) is interfaced with the following elements:

- WIND BRACING	(dwg. No. 510e000)

- ELEVATION PLATFORM	(dwg. No. 512e045)

- FRONT BEAM	(dwg. No. 512e039)

- BACK BEAM	(dwg. No. 512e042/512e011)

- ROLLING SECTOR ‘L’	(dwg. No. 514e004/514e006)

- ROLLING SECTOR ‘R’	(dwg. No. 514e023/514e034)

- CABLE WRAP	(dwg. No. 676a 000/676a001)

- NASMYTH INSTRUMENTS	(dwg. No. 670a025)

- DRIVES AND ‘FIXED GRAVITY INSTRUMENT’	(dwg. No. 670a026)

- ADAPTIVE INSTRUMENTS	(dwg. No. 670a030)

- BALL BEARING FOR INSTRUMENTS	(dwg. No. 670a021)

- BALL BEARING FOR CABLE WRAP	(dwg. No. 670a023)

- PROTECTION FOR GEAR	(dwg. No. 670a025)

- PLANTS INSTALLATION	*

The main above mentioned interfaces are indicated in the drawings listed in dwgs. No. 513e006. All drillings and machining requested in the pertinent interface drawings shall be arranged for.

# 5.0	PERFORMANCE CHARACTERISTICS

The main performance characteristics of the DEROTATORS SUPPORTS are the following:

- rigidly support the DEROTATOR INSTRUMENTS and to keep the alignment of instruments;

# 6.0	RELIABILITY, MAINTAINABILITY REQUIREMENTS

The useful lifetime of the telescope is expected to exceed 25 years.

The contractor shall take into account of all aspects which could limit the lifetime of the supply.

He shall also contain the number of maintenance interventions, supply adequate access for maintenance operations and minimise risks of damage, loss and injury to personnel and the telescope from telescope maintenance operations.

# 7.0	environmental conditions

This chapter defines the natural and induced environmental conditions the telescope and subassemblies will encounter during its lifetime.

- Storage temperature:	-30 to +50°C

- Operating temperature:	-20 to +25°C

- Storage pressure:	500 to 750 Torr

- Operating pressure:	500 to 600 Torr

- Storage humidity:	5 to 80%

- Operating humidity:	5 to 95%

- Maximum operating windspeed:	80 km/h

- Maximum observing windspeed:	120 km/h

- Survival windspeed:	150 km/h

# 8.0	design and construction requirements

The scope of the present chapter is to introduce the main characteristics of the supply from a manufacturing point of view.

The contractor shall take care of confirming the adopted criteria or may propose alternative solutions, in order to obtain the requested performance.

The contractor shall fabricate all parts, equipment, components and materials of the derotators structure  in accordance with the requirements indicated in the present Technical Specification and the applicable drawings.

Subsequent to fabrication, but prior to packaging the materials, the contractor shall store, at its sole expenses, all components, materials and equipment in manner that will reasonably protect such components, materials and equipment against loss or damage.

Suitable allowances for contraction and expansion during welding shall be made in the lengths of the steel parts prior to welding, so that final machined lengths are correct within the acceptance tolerances specified in this Technical Specification and relative drawings. Welding procedures shall be such that distortion and lock-up stresses are minimised. Tack welds, temporary stiffeners, tie bars, controlled peening, pre-heating and stress relieving during welding may be used for this purpose, as approved by P.O..

Surface to be welded shall be dry, clean and free for loose scale, grease, or unsuitable protective paints or coatings. All welded parts and assemblies, unless specified or approved otherwise by P.O., shall be fully stress relieved prior to final machining operations. Stress relieving shall be accomplished by the heat treatment method in a chamber with sufficient volume and temperature capacity to fully stress-relieve the welded parts and subassemblies.

## 8.1	General Dimensions and Mass

The general dimensions of the DEROTATORS STRUCTURE  are shown in dwg. No. 513e006.

Maximum dimensions:	length 6745 mm

- width 3450 mm

- thickness 2190 mm

Total weight:	21659 kg

Lifting points:	t.b.d. with the contractor

## 8.2	Materials

The materials are indicated in the drawings and relative Part Lists.

steel plates:	Fe 510C UNI EN 10025

Steel plates relevant to surfaces to be further machined shall be supplied with complete Ultrasonic Examination Certificates, according to UNI EN 160/87 or ASME SA-578, acceptance standard level I.

## 8.3	Class of Welding

All welding operations shall conform to the standards set forth in American Welding Society Specification AWS D1.1 or equivalent approved by P.O..

Welding operations shall be performed in such a way as to guarantee the following mechanical machining in terms of tolerances, as indicated on drawings. For this purposes, welding procedures shall consider possible shrinkages or expansions during welding operation and minimise the distortion and lock-up stresses.

The contractor shall perform and provide trial welds to demonstrate the soundness of any proposed welding method, and the competence of any person performing welding work, as required by P.O. at any time.

pButt Joints Complete Penetration:

Class I according to UNI 10011, UNI 11001 and UNI 1307/2.

NDT: X-ray or Ultrasonic examination; acceptance criteria as required for above mentioned class of welding, according to UNI 7278, or welding procedures tests according to EN 288-3.

| Welding material | electrode: | according to UNI 5132: Class IVB or according to AWS A5.1: E 7018-1 |
| --- | --- | --- |
|  | wire: | according to AWS A5.20: E 80T5/G |

pCorner or T Joints Complete Penetration:

Class II according to UNI 10011, UNI 11001 and UNI 1307/2.

NDT: X-ray or Ultrasonic examination; acceptance criteria as required for above mentioned class of welding, according to UNI 7278, or welding procedures tests according to EN 288-3.

| Welding material | electrode: | according to UNI 5132: Class IVB or according to AWS A5.1: E 7018-1 |
| --- | --- | --- |
|  | wire: | according to AWS A5.20: E 80T5/G |

pCorner or T Joints Partial Penetration:

Construction: according to UNI 10011, UNI 11001 and UNI 1307/2.

NDT: Magnaflux 60% examination, acceptance criteria category II as per UNI 7278 par.A.4.3.

| Welding material | electrode: | according to UNI 5132: Class IVB or according to AWS A5.1: E 7018-1 |
| --- | --- | --- |
|  | wire: | according to AWS A5.20: E 80T5/G |

## 8.4	General Tolerances

For welded parts: CNR-UNI 10011 and UNI 11001.

Tolerances for linear and angular dimensions are specified on relevant drawings. These tolerances are mandatory, for structure acceptance. Where not specified, ISO 2768 part 1/m and 2/k shall be applied.

Particular attention should be paid to obtain the tolerances requested by the DEROTATORS STRUCTURE after its assembling on the c-ring and elevation platform.

## 8.5	Thermal Treatments

Complete stress relieving.

Parameters: t.b.d. with the contractor.

## 8.6	Tightening Torque of Bolts and Screws

- Metric bolts class 8.8 or 10.9, CNR UNI 10011 paragraph 4.1.3

## 8.7	Surface Treatments

All metallic surfaces, other than rating machining surfaces shall be painted, coated or otherwise permanently protected against atmospheric corrosion.

At a minimum all paints and coatings shall provide for the following functional requirements:

- 25 years service life (with maintenance)

- (two months continuous salt exposure during shipping)

The contractors shall utilise the following paint system in performance of the work.

- sand blasting SA2.5 ISO 8501-1:1988/SS055900:1988

- 1 coat (oil resistant) epoxy polyamide corrosion resistant primer (80 microns)

- 1 coat (oil resistant) vinyl high-build mastic (80 microns)

The supplier can propose an alternative painting cycle; in any case this shall be able to guarantee the above mentioned requirements.

The gears of the rotators bearings and Cable Chain must be protected with grease SKF LGMT 2/5 or other equivalent product.

The surfaces which are machined and not painted shall be protected with protection material of “peeling” type.

## 8.8	Special Manufacturing Instructions

The contractor will present, in a detailed way, the following documents:

- Weldings specifications and procedures: these have to be approved by the Project Office (P.O.) prior starting the work.

- Fabrication plan and control: these have to be approved by the P.O. prior starting the work.

As indicated in chapter 3.0, after machining on every elements the DEROTATORS STRUCTURE shall be coupled each other in order to obtain the indicated tolerances.

Each element of the DEROTATORS STRUCTURE, after fabrication, pre-assembling, welding and machining, shall be checked by means of a measurement system or positioning fixtures, in order to guarantee the positioning of the various coupling flanges and ball bearing for instrument and cable wrap, and the pertinent fabrication tolerances.

Note:	the overmetal indicated on the fabrication of the steel works requires an accurate fabrication (carpentry dimensional tolerances 5÷8 mm). It is up to the contractor to maintain them or to modify them, in order to ensure the successive mechanical machining and relative tolerances. Any modification must be approved by the P.O..

# 9.0	PRE-ASSEMBLY

The DEROTATORS STRUCTURE will be pre-assembled to verify the total dimension and pertinent tolerances indicated in the drawings.

The special tooling required to perform the assembly and the alignment is part of this contract supply. Before disassembling for shipment, all couplings shall be opportunely locked by a pin.

# 10.0	PACKING AND TRANSPORT

All steel structure shall be packed in such a way as to guarantee:

- the degree of the physical-chemical protections against weather conditions during holding and transportations of goods;

- the degree of mechanical protection for the preservation of the goods during handling and transportation to destination;

- the determination of volume and weight in relation to the transportation means.

# 11.0	ACCEPTANCE TESTS

The contractor shall prepare and submit to the P.O. the Acceptance Test Plan.

The following tests and inspections shall be performed prior to Plant Acceptance, and shall be included in the Acceptance Test Plan:

- certificate of raw material and fasteners;

- certification of welding procedure and welding control;

- stress relieve certification;

- dimensional control, machining tolerances, roughness test;

- mass check;

- certificate of surface treatment;

- certificate of assembly tolerances.

# 12.0	SPARE PARTS

Not applicable

# ANNEX A

Doc_info_start

Title: DEROTATORS STRUCTURE SUPPORTS

Document Type: TECHNICAL SPECIFICATION

Source: EIE

Issued by: G.MARCHIORI

Date_of_Issue: 12/01/97

Revised by:

Date_of_Revision:

Checked by:

Date_of_Check:

Accepted by:

Date_of_Acceptance:

Released by:

Date_of_Release:

File Type:MS-WORD 6

Local Name: C0301/DOC/401e031a.doc

Category: TELESCOPE AZIMUTH PLATFORM

Sub-Category: GENERAL TELESCOPE

Assembly: TELESCOPE TECHNICAL DOCUMENTS

Sub-Assembly:DEROTATORS STRUCTURE SUPPORTS

Part Name:

CAN designation: 401e031

Revision: a

Doc_info_end

*	The interfaces with those subsystems are not included in the present Technical Specification and will be prepared in the occasion of the pre-erection.